Exercise duration: ~30 minutes

Practice exercise – Answer Key

# Measure quantities in Takeoff

- The following steps were completed on the videos of Module 2: Perform quantity takeoffs in Forma Takeoff. If you followed along the videos, you are ready to start the exercise. If you have not already performed these steps when following the videos for this module, make sure these steps are completed:

- In the Setting menu of Takeoff, you specified a measurement system and uploaded the Project Level Classification.xlsx into the Classification system 1.

- In the Setting menu of Takeoff, you specified the correct currency and time frame conversions.

- In the Setting menu of Takeoff, you created at least one output definition, for example, an output definition for Window Type A.

- In the Sheets & Models tool:

  - You uploaded the Architectural Model.rvt file from the Version 1 folder in the dataset folder to the 3D Models tab.

  - You uploaded the Architectural Sheets.pdf and Structural Sheets.pdf files from the Version 1 folder in the dataset folder to 2D Sheets tab and extracted the correct sheet names and numbers.

- To start this exercise, create a new takeoff package named Practice Exercise 2.

- Once these steps are completed, you will be ready to start the exercise. You will apply the skills learned in this module to perform quantity takeoff from 2D drawings and 3D models in Takeoff. You will create takeoff types, capture quantities, organize outputs, and use BIM workflows to prepare quantity data for estimating while answering the questions provided.

Learning objectives:

By the end of this exercise, you should be able to:

Perform quantity takeoff from 2D drawings using count, area, and line tools.

Create and organize takeoff types and outputs.

Extract quantities from 3D models using BIM-based takeoff tools.

Create calculated outputs and manage takeoff inventory.

Success criteria:

2D drawing quantities are successfully captured using count, area, and line takeoff tools.

Takeoff types, packages, and outputs are created and configured correctly.

Quantities are successfully extracted from 3D models using BIM-based workflows.

Calculated outputs are created using formulas, and the inventory is updated.

Exercise questions are completed and answers are submitted for review.

- Answer the following questions based on the structural and architectural sheets and the architectural model. Make sure you are working with Version 1. Create a takeoff package named Practice Exercise 2 to put all the quantities in. Create a new BIM-based takeoff type for Single Flush Doors. Add an output definition to Primary line item and use the correct classification for wood doors. - Then using the model tree, find the wooden single flush doors and add them to the takeoff type. How many of these doors are there? - __________________________________________________________________ - Answer Key - There are 15 doors. Using the same takeoff type, add an additional line item for the frames of these doors. The frames are metal and fall under the 08 12 00 classification. Group the inventory by Classification 1 and submit a screenshot as an answer to this question. - __________________________________________________________________ - Answer Key - The student should submit a screenshot of the inventory, which you can also check directly on their project on Forma. Create a new BIM-based takeoff type for ceiling tiles. - In the model, all acoustical ceilings have ACT in their type name, and all ceiling tiles are 2 feet by 2 feet (0.61 m by 0.61 m). Adjust the quantity formula in the Line item details appropriately, then use the model browser to find the ACT ceilings in the model. What is the total number of ceiling tiles required in the model? - __________________________________________________________________ - Answer Key - There are 982 ceiling tiles in the model. - The student should create the following line item details and model browser filters. Create a new BIM-based takeoff type Gypsum wall board to find the number of boards to be purchased. The gypsum boards are 8 feet by 4 feet (2.44 m by 1.22 m). - In the model, gypsum boards are used in walls of type mark E1 and E4 and in some ceilings that have GWB in their type name. What is the total number of gypsum boards needed in the model? - __________________________________________________________________ - Answer Key - The number of gypsum wall boards is 706. - The student should create the following line item details and model browser filters. Using sheet S103, what is the quantity of cast in place concrete needed, in cubic yards (cubic meters), for spread footing F1 which are 1’6’’ deep (0.46m)? You should create a new area-based takeoff type named Footing F1 and the Concrete Cast-In-Place from the Hub Library. - __________________________________________________________________ - Answer Key - There quantity of concrete is about 30 CY (~23 cubic meters). - The student should create the following takeoff type.