# Exercise duration: ~20 minutes

# Practice exercise

# Load and apply classification information

Load a classification system into a Revit project model. Read information requirements documentation before assigning the necessary information to the 3D model.

Learning objectives:

- Load classification systems into a Revit project.

- Read information requirements documentation.

- Apply information to 3D model elements within a Revit project model.

- Apply project information to a Revit project.

- Investigate information attached to elements and contained within a Revit project.

The completed exercise

| In Revit, open the provided dataset, Base level complete.rvt. | Figure 1. Revit project file for the exercise. |
| --- | --- |
| Select all of the external walls. | Figure 2. All external walls selected. |
| Click Assign Classification to open the classification dialog. | Figure 3. Interoperability Tools > Standardized data panel. |
| Load the Uniclass 2015 database into the project. | Figure 4. Standardized Data dialog. |
| Open the Air Appendix A.xls file to view the information required. - For this example, it is stage 3 and you work for the Architectural Company. - You have walls and doors in your model, and they all require information at this stage. | Figure 5. AIR Appendix A. |
| External walls require System information indicated by 3A. - In the Classification dialog, click to change the table from EF to System Ss. | Figure 6. External wall classification. |
| Expand to find the table and classification code as per the AIR appendix. | Figure 7. Standardized Data dialog. |
| Select the code from the list and click Assign to assign the code to all of the external walls. - The code should highlight blue for a moment and display the word ASSIGNED. | Figure 8. Wall Uniclass information assigned. |
| In the 3D view, click Edit Type to inspect the type properties of the External Walls. - You should see the code as per the AIR appendix. | Figure 9. Properties palette. |
| Select the internal walls. | Figure 10. All internal walls selected. |
| Internal, or partition walls require System information indicated by 3A. | Figure 11. AIR appendix. |
| Open the Classification dialog. | Figure 12. Assign classification. |
| Assign the classification code to the internal walls as per the AIR appendix. | Figure 13. Classification codes. |
| Select all of the doors in your project. | Figure 14. All doors in the model selected. |
| Doors require System and also Type information to be assigned during stage 3. | Figure 15. Door Uniclass information. |
| Assign both codes to all doors. | Figure 16. Classification information. |
| In the 3D view, ensure no elements are selected and click Assign Classification to open the dialogue. | Figure 17. Uniclass tables. |
| Expand the codes to find En_45_10_39 and click Assign to assign it to the project. | Figure 18. Uniclass Entity codes |
| In the Ribbon, Manage tab, Settings panel, click Project Information to open the dialog. | Figure 19. Manage tab > Settings panel >Project Information. |
| Add the information below here as per the BIM Execution Plan. - Status. - Client. - Project name. - Address. | Figure 20a. Project Information dialog. Figure 20b. BIM Exécution Plan. |