Exercise duration:  ~90 minutes

Project challenge exercise – grading rubric

# Create foundation walls, footings, and rebar in Revit

With the provided Revit models, 03_ARCHITECTURAL.rvt, and the template Structural Analysis-DefaultMetric.rte. use your skills to solve this project challenge.

Scenario:

As the structural designer on a project, you will use the linked architectural model as a guide to create a 457mm concrete foundation wall and footing. Then you’ll add horizontal and vertical reinforcing.

Complete the required activities:

Start a new structural model using the Structural Analysis-DefaultMetric.rte template.

Link the model 03_ARCHITECTURAL.RVT model using the Internal Origin-to-Internal Origin placement.

Set the Discipline parameter in Level 1 to Coordination.

Create a T.O.FOOTING level at -3084 mm below Level 1. Add a 457 mm foundation wall extending from Level 1 to the top of footing level. Add this to the entire exterior of the building.

In the top of footing level, add a bearing wall footing to the base of the new foundation wall.

Duplicate the existing bearing footing type in the model and create: Bearing Footing - 762 x 305. Once the footing type is created, select the perimeter walls to place it under.

Add a section to the wall once the bearing footing has been placed. Set the scale to 1:25. Set the Detail Level to Fine.

Add a keyway by using the Cut Profile tool and selecting Boundary between faces option. Make the keyway 4” x 2”.

Add 19 mm rebar perpendicular to cover at a spacing of 305 mm.

Add 19 mm rebar parallel to cover at a spacing of 305 mm.

Success Criteria:

Model consistency: The architectural model is linked and at the internal origin of the model. The foundation wall is from Level 1 to Top of Footing.

Concrete elements sized properly: The foundation wall is the correct thickness, the keyway is the correct size, and the footing is the correct size and thickness.

Correct reinforcing is added: The reinforcing is the correct size and spacing. The reinforcing is in the correct planar direction.

What to Submit:

Revit model file (.RVT) – The structural model with the foundation walls, footings and rebar completed.

Grading Rubric

|  | Advanced | Proficient | Basic | Emerging |
| --- | --- | --- | --- | --- |
| Linking the architectural model | The architectural model is linked to the internal origin. In the properties, the discipline is set to Coordination. | The architectural model is displayed. In the properties, the discipline is set to Coordination. | The architectural model is displayed. In the properties, the discipline is set to Structural. | The architectural model is linked center-to-center |
| Create the foundation wall | The foundation wall is modeled with the correct top and bottom constraints. The wall is the correct thickness and | The foundation wall is modeled with the correct top and bottom constraints. The wall is the default thickness, | The foundation wall is modeled from level 1 with an offset of 4572mm. | The foundation wall is a generic wall. |
| Add the footing | The footing is sized with 153mm thicker than the bearing wall. The thickness is 305mm. | The footing is sized with 153mm thicker than the bearing wall. The thickness is 610mm. | The footing is a retaining footing. | No footing has been placed. |
| Creating the keyway | The cut profile tool was used. The keyway is at the specified dimensions. | The cut profile tool was used. The keyway is not drawn at the correct dimensions. | The cut profile tool was used. The keyway was cut but not using the between two elements tool. | There is no keyway added to the footing. |
| Adding horizontal bars | Bars have been placed perpendicular to cover with a maximum spacing of 305mm. The rebar is against the cover settings in the wall. The rebar is the correct size and shape. | Bars have been placed perpendicular to cover with a maximum spacing of 305mm. The rebar is against the cover settings in the wall. The rebar is the incorrectly sized. | Bars have been placed perpendicular to cover without the correct spacing and size. | No bars have been placed. |
| Adding Vertical bars | Bars have been placed parallel to cover with a maximum spacing of 305 mm. The rebar is against the cover settings in the wall. The rebar is the correct size and shape. | Bars have been placed parallel to cover with a maximum spacing of 305 mm. The rebar is against the cover settings in the wall. The rebar is the incorrectly sized. | Bars have been placed parallel to cover without the correct spacing and size. | No bars have been placed. |